PAGE  
13
[image: image1.jpg]We Suppont

EEEEEEEEE





Vacuum Testing 
Meets NATEF Task:  (A8-A-8)  Perform engine absolute (vacuum/boost) manifold pressure tests; determine necessary action. (P-1)
[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1                    
_____  1.  Connect the vacuum gauge to a manifold vacuum source (source of vacuum at idle).

_____  2.  Vacuum at idle = _____ in. Hg.   (should be 17-21 in. Hg. and steady).

_____  3.  Drive the vehicle on a level road in high gear at a steady speed.



Cruise vacuum = _____ in. Hg.    (should be 10 - 15 in. Hg.)

_____  4.  Accelerate the vehicle in high gear to W.O.T.



W.O.T. vacuum = _____ in. Hg.    (should be almost zero)

_____  5.  Decelerate the vehicle from 50 MPH with the throttle closed.



Deceleration vacuum = _____ in. Hg.    (should be higher than idle vacuum)

_____  6.  With the engine out of gear and the brake firmly applied, raise the engine speed to 

                 2,000 RPM and hold for one full minute.  This tests for an exhaust restriction.



Results = ______________ in. Hg.    

_____  7.  Stop the engine.  Disable the ignition.  Crank the engine and observe the vacuum 

                 during cranking.



Cranking vacuum = _______ in. Hg.    (should be higher than 2.5 in. Hg.)



OK_____
NOT OK_____

_____  8.  Based on the vacuum test results, what is the necessary action? __________________



__________________________________________________________________


