PAGE  
15
[image: image1.jpg]We Suppont

EEEEEEEEE





Compression Testing
Meets NATEF Task:  (A8-A-10)  Perform cylinder cranking and running compression tests; determine necessary action. (P-1)
[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1                  
_____  1.  Remove all spark plugs (be certain to label the spark plug wires) and disable the 


     ignition system to avoid possible ignition coil damage.

_____  2.  Block open the throttle and choke (if equipped).

_____  3.  Crank the engine at least 4 “puffs” (compression strokes) while observing the gauge.

NOTE:  For accurate test results, the engine should be at normal operating

temperature.

RESULTS:  1st puff / final reading      
1st puff / final reading



 1.  ___________/__________  

5.  ___________/__________     



 2.  ___________/__________  

6.  ___________/__________


   
 3.  ___________/__________  

7.  ___________/__________



 4.  ___________/__________  

8.  ___________/__________

NOTE:  The 1st "puff" should be more than one-half of the pressure of the final 

puff.  If the 1st puff is low, worn piston rings are likely – repeat the test.

_____  4.  Reinstall all spark plugs except one.  Perform a running compression test at idle and at 


     2000 RPM for each cylinder:


           

Idle                           

2000 RPM


1. ________________________     1.  ________________________


2. ________________________     2.  ________________________


3.  _______________________      3.  ________________________


4.  _______________________      4.  ________________________


5.  _______________________      5.  ________________________


6.  _______________________      6.  ________________________

7.  _______________________     7.  ________________________

8.  _______________________     8.  ________________________

OK_____
NOT OK_____

_____  5.  Based on the test results, what is the necessary action? _________________________


     _____________________________________________________________________


