PAGE  
69
[image: image3.jpg]




Throttle Position Sensor Scope Test
Meets NATEF Task:  (A8-B-7) Inspect and test sensors, actuators, and circuits using a graphing multimeter (GMM)/digital storage oscilloscope (DSO); perform necessary action.  (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1              
[image: image2.jpg]FLLIKE 98 AUTOMOTIVE SCOPEMETE

-1V

POTENTIOMETER

RECORD




_____  1.  Check service information regarding the 

     location of the throttle position sensor

     used on the vehicle being tested.


Location (describe):  

_____________________________


_____  2.  Check service information for the wire colors used and their purpose

Wire 1 (color and purpose): 
_________________________           

Wire 2 (color and purpose): 
_________________________


Wire 3 (color and purpose): 
_________________________


[image: image3.jpg]_____  3.  Check service information for the specified voltage 

     output and/or waveform for the throttle position sensor.

 ________________________


_____  4.  Following the test equipment manufacturer’s 

     instructions, show the instructor the waveform.







Instructor OK _______

_____  5.  Based on the comparison between the captured 

     waveform and the specified waveform, what action 

     is needed?

______________________________________________________________

______________________________________________________________

