PAGE  
73
[image: image2.jpg]




Throttle Position Sensor Resistance Test

Meets NATEF Task:  (A8-B-7) Inspect and test sensors, actuators, and circuits using a graphing multimeter (GMM)/digital storage oscilloscope (DSO); perform necessary action.  (P-1)

[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1                      
_____  1.  Check service information for the specified testing procedures and resistance values of 


     the throttle position sensor.



__________________________________________________________________



__________________________________________________________________

_____  2.  After disconnecting the electrical connectors, measure the resistance of the TP sensor 


     using a digital multimeter (DMM) set to read ohms (Ω).



a.  Measure the resistance between the terminals for the 5-volt reference and the 



     signal resistance (ground) terminal.




_________ (should be about 1000 ohms (0.8 to 1.1 KΩ)




____ OK    ____ NOT OK


b.  Measure the resistance between the 5-volt reference terminal and the signal 



     wire.




_____ ohms



c.  Depress the accelerator and observe the meter.  Does the resistance increase or 



     decrease?  Why?




____________   ______________________________________________




____________________________________________________________

[image: image2.jpg]_____  3.  Based on the tests performed, what is the necessary action? _____________________



_____________________________



_____________________________



_____________________________

