PAGE  
78
[image: image1.jpg]We Suppont

EEEEEEEEE





MAP Sensor Output Chart

Meets NATEF Task:  (A8-B-7) Inspect and test sensors, actuators, and circuits using a graphing multimeter (GMM)/digital storage oscilloscope (DSO); perform necessary action.  (P-1)

[image: image1.jpg]Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1                   
Most vehicle MAP sensors use a 5-volt reference, a signal return and a ground connection.  As the vacuum inside the intake manifold changes, the MAP sensor voltage changes.

· High vacuum (low absolute manifold pressure) = low voltage

· Low vacuum (high absolute manifold pressure) = high voltage

_____  1.  Connect a T-pin to the signal wire connector at the MAP sensor.

_____  2.  Connect the red lead of a digital voltmeter set to read DC volts to the T-pin.  Connect 

                 the black meter lead to a good chassis ground.

_____  3.  Disconnect the vacuum hose from the MAP sensor (or remove the sensor from the 

                 manifold) and attach a hand-operated vacuum pump to the sensor.

_____  4.  Place a dot on the graph that represents the MAP sensor voltage when 5, 10, 15, 20, 25 

                 and 30 in. Hg of vacuum is applied to the sensor.

_____  5.  Connect the dots.  The results should be a straight line.



OK _____    NOT OK _____

	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	


          0                  5                  10                  15                 20                 25                 30
                                                                VACUUM APPLIED (in. Hg)

_____  6.  Based on the results of this test, what is the necessary action? ___________________



_________________________________________________________________
