PAGE  
90
[image: image2.jpg]




Fuel Pump Current Ramping
Meets NATEF Task:  (A8-D-3)  Inspect and test fuel pumps and pump control systems for pressure, regulation, and volume; perform necessary action. (P-1) 
[image: image1.jpg]We Suppont

EEEEEEEEE



Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1                   
_____  1.  Check service information for the specified fuel pump operating speed (normally from 

[image: image2.jpg]     3500 to 7500 RPM). 

______________________________________

_____  2.  Tools and equipment needed to determine 

  
     the rotational speed of the fuel pump include:


____  Digital storage oscilloscope


____  Current clamp


____  Fused jumper lead


_____  3.  Perform a current ramping procedure and determine the following:



____  Number of commentator segments = ______ (usually 8, 10, or 12)



____  Milliseconds needed for one rotation of the pump = ______



____  Fuel pump speed = 60,000 ÷ milliseconds = _______

_____  4.  Based on the test performed, what is the necessary action?  ___________________



_______________________________________________________________



_______________________________________________________________

