Front Shock Absorber Replacement

Meets ASE Task:  (A4-D-1) Inspect, remove, and replace

shock absorbers.  (P-1)

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

_____  1.  Verify that the front shock absorber requires replacement.  Check all that apply:

_____ bent or damaged shock or mounting hardware

_____ shock absorber is leaking hydraulic fluid

_____ excessively worn - causing tire wear or riding comfort problems

_____ other (specify) _______________________________________________

_____  2.  Compare the replacement shocks to the original shocks to be sure that they are correct.

OK _____     NOT OK _____

NOTE:  All shock absorbers should be replaced in pairs only.  Do not replace just one shock absorber.

_____  3.  Check the service information for the specified replacement procedure. ____________

__________________________________________________________________

__________________________________________________________________

HINT:  Many shocks on rear-wheel-drive vehicles can be broken off using a

deep-well socket and a long extension.  By rocking the extension back and forth, the top of the shock will usually break off saving the time and effort it takes to remove a nut that is often rusted in place after many years of service.

_____  4.  Safely hoist the vehicle.

_____  5.  Remove the lower shock absorber retaining bolts (nuts) as per the service information

instructions..

CAUTION:  Be ready to catch the shock absorber because it will likely fall after removing the last retaining bolt (nut).

_____  6.  Show the instructor the removed shock absorber.  Instructor’s OK _____________

_____  7.  Extend the rod on the replacement shock and install the lower retaining bolts (nuts).

_____  8.  Lower the vehicle and install the upper retaining fastener.

_____  9.  Bounce the vehicle to check that the replacement shock does not interfere with any

part of the suspension or frame.

_____ 10. Test drive the vehicle before returning it to the customer.