Rear Disc Parking Brake Adjustment

Meets ASE Task:  (A5-D-10) P-2  Adjust calipers with integrated parking brake.

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

Many vehicles equipped with rear disc brakes use a mechanical activated parking brake that is integral with the caliper.  Most are designed to be self-adjusting by adjusting when excessive brake pad-to-rotor clearance occurs.

_____  1.  Check the service information for the specified rear disc brake parking brake

adjustment procedure.

_________________________________________________________________

_____  2.  Check the number of “clicks” of the parking brake.

_____ Number of clicks (should be between 3 and 9)

OK _____     NOT OK _____

If over 10 clicks is needed to set the parking brake, the rear disc brake caliper needs

adjustment.

_____  3.  Hoist the vehicle safely and remove both rear wheels.

_____  4.  Carefully inspect the rear disc brakes for damage and

measure the pads for excessive wear.

OK _____     NOT OK _____

Replace the pads if worn to the minimum

allowable thickness.

_____  5.  If the disc brake pads are serviceable, operate the parking brake lever using the

appropriate size wrench on the actuating arm retaining bolt/nut while lightly tapping

on the caliper using a dead blow plastic hammer.  The adjusting mechanism should

cause the piston to be repositioned with the correct pad to rotor clearance.

OK _____     NOT OK _____

If the proper clearance is not achieved, replacement of the calipers is required.