Vacuum Power Brake Booster Test

Meets ASE Task:  (A5-E-1) P-2  Test pedal free travel; check power assist operation.

Name _______________________________   Date ___________  Time on Task __________

Make/Model/Year _________________   VIN ________________   Evaluation:   4    3    2    1

_____  1.  Check the service information for the specified procedure for testing a vacuum power

brake booster for the vehicle being serviced.

__________________________________________________________________

_____  2.  With the engine off, depress the brake pedal several times until the brake pedal feels

hard (firm).

_____  3.  The brake pedal should not fall to the floor of the vehicle.

OK _____     NOT OK _____

NOTE:  If the brake pedal travels to the floor

of the vehicle, carefully inspect the hydraulic

brake system for a fault.  Service or repair the

hydraulic brake problem before continuing

with this test.

_____  4.  With your foot still firmly depressing the brake

pedal, start the engine.  The brake pedal should

go down.

OK _____     NOT OK _____

_____  5.  If the brake pedal did not go down when the engine was started, visually check the

following:

_____ Minimum of 15 in. Hg of vacuum to the vacuum booster from the engine

manifold or auxiliary vacuum pump

_____ Proper operation of the one-way check valve

_____ Unrestricted charcoal filter between the booster and the intake manifold (if

equipped)

_____ Inspect for vacuum leaks

OK _____     NOT OK _____