Evaluation (Enter number from 4, 3, 2, 1) :_________

Meets ASE Task: Not specified.

Time on Task:_____________

Make/Model/Year:____________________________________

VIN:_________________________________

Date:_____________

Name:_____________________________________

Alternator Rotor Testing

1.	Carefully inspect the rotor for damage. 	OK _____	NOT OK _____

2.	Use 400 grit emery cloth to clean the slip rings. Be sure to rotate the slips in the cloth to avoid creating flat areas.

3.	Set a digital multimeter (DMM) to read ohms (low scale).

4. 	Measure the resistance between the slip rings and compare with specifications:

actual = __________		OK _____	NOT OK _____

GM = 2.2 to 3.5  		Ford = 3.0 to 5.5       Chrysler = 3.0 to 6.0

5.	To test that the rotor winding is not shorted-to-ground, place one meter lead on a slip ring and the other meter lead to the steel shaft of the rotor.  The reading should be infinity (OL) if the rotor is OK.

reading = _____________		OK _____	NOT OK _____