Evaluation (Enter number from 4, 3, 2, 1) :_________

Meets ASE Task: A5 – G-6 – P-2

Time on Task:_____________

Make/Model/Year:____________________________________

VIN:_________________________________

Date:_____________

Name:_____________________________________

Depressurization of High-Pressure ABS

1.	Integral ABS systems combine the function of the master cylinder, power-assist booster, and antilock brake functions in one assembly.  These assemblies operate at high pressure and must be depressurized before performing service work on the brake system to avoid possible personal injury.

2.	Check the service information for the specified depressurization procedure for the vehicle being serviced.

______________________________________________________________________________

______________________________________________________________________________

3.	Visually check the brake fluid reservoir.

Proper level?		OK _____     NOT OK _____

Brake fluid condition?  Describe: ______________________________________

4. 	Inspect the ABS hydraulic control unit for signs of damage or leakage.

OK _____     NOT OK _____

5.	With the ignition key off, depress the brake pedal forty (40) times.  The brake pedal should be hard when depressed after the first few brake applications.

OK _____     NOT OK _____

If the brake pedal is not hard and a power-assisted brake application is still possible, find and correct the ignition feed circuit to the hydraulic control unit before proceeding to brake system service.