Evaluation (Enter number from 4, 3, 2, 1) :_________

Meets ASE Task: A4 – B-8 – P-2

Time on Task:_____________

Make/Model/Year:____________________________________

VIN:_________________________________

Date:_____________

Name:_____________________________________

Tie-Rod End Inspection and Replacement

1.	Verify that the tie-rod end(s) requires replacement.  Check all that apply.

Torn boot.

Joint has side-to-side movement.

Physically damaged

Other (describe): _______________________________________________________

2.	Hoist the vehicle safely.

3.	Compare the replacement tie-rod end with the original to be sure that the new end is correct.

4. 	Remove the retaining nut and use a tie-rod puller to separate the tie-rod end from the steering knuckle and/or center link.

5.	Measure the distance between the center of the tie-rod end and the adjusting sleeve and record this distance so the replacement tie-rod end can be installed in approximately the same location so that the wheel alignment (toe setting) will be close to being correct.

6.	Unscrew the old tie-rod end and discard.

7.	Install the replacement tie-rod end and adjust to the same distance as measured and recorded in #5.

8.	Install the tie-rod end onto the steering knuckle and torque the retaining nut to factory specifications.

Torque specifications for the tie-rod retaining nut = ___________

9.	Lower the vehicle and align the vehicle before returning it to the customer.