Evaluation (Enter number from 4, 3, 2, 1) :_________

Meets ASE Task: Not specified.

Time on Task:_____________

Make/Model/Year:____________________________________

VIN:_________________________________

Date:_____________

Name:_____________________________________

Alternator Stator Testing

1.	Identify the type of stator.

_____ Wye (has three terminals with one wire at each terminal and a wire junction)

_____ Delta (has three terminals with two wires at each terminal)

2.	Visually inspect the stator for faults such as burned insulation due to overheating or broken wires.

OK _____	NOT OK _____

3.	Set a digital multimeter to read ohms (low scale).

4. 	Measure the resistance between all three terminals of the stator two at a time.  The resistance will be low (usually less than 2 ohms). If high or infinity, the stator is defective.

OK _____	NOT OK _____

5.	To check if the stator windings are shorted-to-ground, connect one lead of the meter (still set to read ohms) to the steel laminations of the stator and touch the other lead to each of the three terminals.  The reading on all three terminals should be infinity (OL).

OK _____	NOT OK _____